Day 2 Task

Both switches have an empty MAC address table, and all PCs have an empty ARP table.

- If PC1 pings PC3, what messages will be sent over the network, and which devices will receive them? 
ARP Request (received by PC2, PC3, PC4) broadcast message
ARP Reply (received by PC1) unicast message
PC1 add PC3 mac address to its ARP Table and use it as destination address
ICMP Echo request from PC1 to PC3 (unicast message received by PC3 only)
ICMP Echo reply from PC3 to PC1 (unicast message received by PC1 only)

- Send the ping and use Packet Tracer’s simulation mode to verify your answer. 
ping 192.168.1.3

- Use pings to generate network traffic and allow the switches to learn the MAC addresses of all PCs on the network. 
ping 192.168.1.4

- Use show commands on the switches to identify the MAC address of each PC. 
sh mac-address table

- Clear the dynamic MAC addresses from the MAC address table of each switch.

clear mac-address-table dynamic

🚀 Day 2 of My 60-Day Network Engineering Challenge

Today, I explored how devices communicate on a network when nothing is known yet — no MAC addresses, no ARP entries.

Here’s what I worked on:

- Simulated communication between PCs with empty MAC & ARP tables

- Observed how ARP requests are broadcast across the network

- Used Packet Tracer simulation mode to trace packet flow

- Generated traffic to allow switches to learn MAC addresses

- Verified learned MAC addresses using show commands

- Cleared MAC address tables to reset learning

💡 Key Takeaways:

- ARP requests are broadcast to all devices in the network

- ARP replies are unicast back to the sender

- Switches learn MAC addresses dynamically from incoming frames

- MAC address tables are built automatically through traffic

⚠️ What stood out:
At the start, the switch behaves like a hub (flooding traffic) until it learns MAC addresses.

📸 (Attach Packet Tracer simulation screenshot)

This is how networks actually learn and adapt in real time.

✅ Day 2 done
🎯 58 days to go

#CCNA #NetworkEngineering #Cisco #LearningInPublic #Networking #TechJourney #PacketTracer